Effects of modifier cations on halide behavior in sulfate-containing peralkaline aluminoborosilicate glasses

Rodman, K.L.* (1), Bussey, J.M. (1), Soudani, S. (1), Dixon Wilkins, M.C. (1), McCloy, J.S. (1).

(1) Washington State University, USA

* Lead presenter

Volatile anions (such as SO42- F-, Cl-, and I-) present challenges for nuclear waste vitrification due to their low incorporation in borosilicate glass and subsequent loss as water soluble or volatile phases. It has been shown that factors such as melt composition, compensator species, and synthesis conditions impact halide incorporation and salt formation. In current work, we employ X-ray absorption (XANES) and nuclear magnetic resonance (19F, 35Cl, 11B, 23Na, and 27Al MAS NMR) spectroscopies to investigate the coordination environment of halides (F, Cl, and Br) in simulant Hanford site peralkaline aluminoborosilicate glasses with varying modifier ratios (Na:Ca). Preliminary results confirm that halide incorporation is limited both by site preference and availability of modifier cations, and that these limitations are different among the halides. Halide speciation and environment, preferential incorporation methods, and implications for vitrification systems will be discussed.