Understanding of the Behaviour of Ruthenium in the Vitrification of Highly Active Waste

Bibi Shehrbano (1), Colin Boxall (1), Joshua Turner (2), Richard Wilbraham (1).

Affiliations (use a semicolon to separate authors)

(1) Lancaster University, Lancaster, United Kingdom

(2) United Kingdom National Nuclear Laboratory (UKNNL), Sellafield, United Kingdom

*Bibi Shehrbano

Abstract

The recycling of spent nuclear fuel enables the recovery of actinides such as uranium and plutonium but also generates highly active liquid waste that requires safe treatment and immobilisation. Ruthenium is a key concern due to radioactive Ru106 and its tendency to form volatile species during high-temperature vitrification. In this study, RuO2 was examined under oxidising conditions to understand its thermal behaviour. Thermogravimetric analysis of commercial RuO2 under nitrogen, air, and oxygen showed an initial mass loss due to dehydration, followed by further mass loss under oxidising conditions, indicating volatilisation via formation of RuO4. Following TGA, SEM and XRD analyses showed recrystallisation and grain growth, especially in inert and air atmospheres. These results highlight the influence of atmosphere and temperature on RuO2 stability and volatility. These findings can provide improved strategies to limit ruthenium release during high-level waste vitrification.