Title of abstract: Rate resumption in UK Ca/Zn waste glasses from gel layer disruption by secondary phases

Authors (use a comma to separate authors, underline presenting author)

Fisher, A.J.(1, 2), Painter, P.R.W.*(3), Townsend, L.(1), Corkhill, C.L.(3)

Affiliations (use a semicolon to separate authors)

(1) NucleUS Immobilisation Science Laboratory, Department of Materials Science & Engineering, The University of Sheffield, Mappin Street, Sheffield S1 3JD, UK

(2) Sellafield Ltd., Sellafield, Seascale, Cumbria CA20 1PG, UK

(3) School of Earth Sciences, University of Bristol, Queen's Road, Bristol, BS8 1RJ, UK
* lead presenter

Abstract (max length: 1000 characters including spaces – only plain text – no figures – including references)

Calcium and zinc are added to UK high level waste glass to aid the waste loading of Mo. The addition of Zn is known to affect aqueous durability; while it confers increased short-term durability, it may enhance long term dissolution due to the precipitation of Zn-bearing secondary phases. We propose a novel mechanism whereby such secondary phases kinetically transform within the gel layer, destabilising the gel and causing a resumption of dissolution. We show, using micro-focus EXAFS, that Zn in the gel layer converts with time from a hardystonite- (Ca2Zn2Si2O7) like coordination environment to a willemite (Zn2SiO4)-like environment, with formation of nano-crystalline ZnO2 the eventual stable phase. This crystallisation behaviour of Zn-bearing phases within the gel is hypothesised to lead to increased porosity and mechanical failure, increasing the access of the solution to the pristine glass and the onset of rate resumption.