A Thermodynamic Analysis of Effect of Temperature on Swelling Stress of Ca-Bentonite

Kosuke Ichikawa*(1), Haruo Sato(1)

(1) Graduate School of Environmental, Life, Natural Science and Technology, Okayama University, Japan

* lead presenter

Buffer material, one of the engineered barrier elements in the geological disposal of a high-level radioactive waste, develops swelling stress due to groundwater penetration from the surrounding rock mass. The swelling stress of bentonite may be affected by the temperature increase caused by decay heat. The authors developed a thermodynamic model to calculate the temperature dependence of bentonite swelling stress using thermodynamic data for interlayer water in Na- and Ca-montmorillonite.

For Ca-montmorillonite, equilibrium relative humidity was measured at fixed water contents over a range of temperatures. The relative partial molar enthalpy was derived from the obtained data. Using the obtained enthalpy and the Gibbs free energy at standard temperature (298.15K), swelling stress was calculated at arbitrary temperatures. The results suggest that swelling stress increases slightly with increasing temperature.