Title of abstract: Sequential THM Coupling of PFLOTRAN and Code_Aster for Performance Assessment of Geological Disposal Systems

Authors

Lee, H.J.*(1), Park, C.H.(1), Park, J.Y.(1)

Affiliations

(1) Ulsan National Institute of Science and Technology (UNIST), South Korea
* lead presenter

Abstract

Evaluating geological disposal systems requires Thermal-Hydrological-Mechanical-Chemical (THMC) modeling. PFLOTRAN (FVM) excels in THC processes, radioactive decay, and waste package functions but lacks robust mechanics (M) capabilities. To address this, a sequential framework was developed linking PFLOTRAN with Code_Aster (FEM), leveraging FVM and FEM strengths for efficient THMC coupling. Advanced geomechanical models were integrated into Code_Aster: the BBM/BExM for bentonite swelling and a copper creep model, establishing a flexible foundation that enables continuous integration of newly derived experimental material models. The framework is applied to evaluate stresses in the host rock and engineered barriers, optimize design, and assess rock fractures under seismic scenarios, providing an integrated tool for the performance assessment of geological disposal systems.